SELF-TIMED RECURRENT THRESHOLD-LOGIC NETWORK WITH CLOCKLESS SEQUENCING OF GRAPH-COLORED NODE PARTITIONS

Inventors: Bill Lin, San Diego, CA, US; Aarav Vidhawan, San Diego, CA, US

Assignee: [The Regents of the University of California, Oakland, CA, US]

Attorney/Agent Docket No.: [TO BE ASSIGNED BY UC SAN DIEGO OFFICE OF INNOVATION AND COMMERCIALIZATION]

CROSS-REFERENCE TO RELATED APPLICATIONS

[0001] Not applicable.

STATEMENT REGARDING FEDERALLY SPONSORED RESEARCH OR DEVELOPMENT

[0002] Not applicable.

FIELD

[0003] The present disclosure relates generally to electronic circuits, and more particularly to asynchronous (clockless) digital logic circuits implementing recurrent networks of threshold-logic nodes, including circuits for content-addressable associative memory.

BACKGROUND

[0004] Recurrent networks of binary threshold units, of which the Hopfield network is a well-known example, converge to stable fixed points (attractors) of an associated energy function when their units are updated one at a time. Such networks are useful as content-addressable associative memories, in which a corrupted or partial input pattern is restored to a stored pattern, and as solvers for combinatorial optimization problems, in which coupling weights encode a cost function whose low-energy states correspond to good solutions.

[0005] Convergence guarantees for such networks generally assume serialized updates. When many mutually coupled units are instead updated simultaneously, as in a fully parallel synchronous implementation, the network state can enter persistent oscillations rather than settling to a fixed point. Serialized updating avoids oscillation but sacrifices parallelism and requires update-scheduling machinery, typically including a global clock, address sequencing logic, and state registers, all of which consume area and power and constrain throughput.

[0006] Partitioned update scheduling based on graph coloring has been employed in clocked implementations, including implementations in which distinct phase-shifted periodic clock signals are supplied to distinct color groups of stochastic units. Such implementations retain clock generation and distribution circuitry and rely on open-loop timing margins fixed by the clock phases and periods. Conversely, sequencerless stochastic implementations have been described in which units update autonomously at random times without any enforced ordering; such implementations provide no structural guarantee against coupled units acting on stale values and lack deterministic convergence guarantees.

[0007] Clocked digital implementations additionally expend energy in clock distribution and in registering network state on every cycle, including cycles in which few or no units change state. As a network approaches an attractor, most update cycles are wasted work. There is accordingly a need for circuit architectures that retain the convergence behavior of serialized updating while permitting substantial parallelism, and that avoid the overheads of global clocking.

SUMMARY

[0008] Aspects of the present disclosure provide a self-timed circuit implementing a recurrent network of Boolean threshold-logic node circuits, in which the node circuits are partitioned into update classes corresponding to independent sets of the network's interaction graph (a proper vertex coloring), and in which sequencing circuitry enforces, without any global clock signal, an update ordering among the update classes such that the node circuits of one update class settle to stable values before node circuits of another update class respond to them. Because no two node circuits within an update class are directly coupled, all node circuits of a class may evaluate concurrently without interacting, and the enforced ordering between classes renders the aggregate evolution equivalent to a serialized (block-sequential) update schedule, for which convergence to a fixed point is guaranteed for symmetric coupling weights. The architecture thereby obtains the parallelism of class-wide concurrent evaluation, the convergence guarantee of serial updating, and the power and area advantages of clockless operation, in which switching activity is proportional to distance from convergence and a settled network dissipates substantially no dynamic power.

[0009] In some embodiments, the sequencing circuitry comprises delay circuitry establishing different signal propagation delays for different update classes, such that a slower class responds to the outputs of a faster class only after the faster class has settled. In other embodiments, the sequencing circuitry comprises completion detection circuitry associated with each update class and handshake circuitry, for example comprising one or more Muller C-elements, that enables evaluation of a subsequent update class in response to assertion of a completion signal by a preceding update class, providing an ordering enforced by causality rather than by timing margin and therefore robust to process, voltage, and temperature variation. In some embodiments the delay circuitry comprises inertial delay elements that suppress pulses shorter than a rejection window, whereby hazard-induced glitches arising within a node circuit are prevented from propagating to other node circuits without requiring hazard-free logic synthesis. In some embodiments the handshake circuitry comprises a state-holding phase element, such as a generalized Muller C-element, having a set function responsive to completion of one update class and a reset function responsive to completion of another update class, the update-class enable signals being derived from the state of the phase element.

[0010] In some embodiments, convergence detection circuitry asserts a convergence signal upon determining that a complete pass through all update classes has produced no change in any node output, providing an intrinsic, zero-overhead indication that the network has reached an attractor. In some embodiments the number of update classes is exactly two and the interaction graph is bipartite, either because the coupling topology is constrained to be bipartite during training or because each network variable is represented by a pair of duplicate node circuits coupled by an agreement-enforcing weight, whereby any interaction graph is rendered bipartite. In some embodiments the coupling weights are binary or ternary and each threshold function is evaluated at least in part by a population count circuit, substantially reducing the logic cost of each node circuit. Further embodiments include methods of operating such circuits and methods of producing such circuits from a trained recurrent network, including retraining coupling weights restricted to a sparse connectivity support.

[0011] The foregoing summary is illustrative only and is not intended to be limiting. Additional aspects, embodiments, and features are described below.

BRIEF DESCRIPTION OF THE DRAWINGS

[0012] FIG. 1 is a block diagram of a self-timed recurrent threshold-logic network circuit in accordance with some embodiments, showing a plurality of node circuits, a recurrent interconnection network, sequencing circuitry, and convergence detection circuitry.

[0013] FIG. 2 illustrates an example interaction graph partitioned into update classes by proper vertex coloring, in which no two nodes of the same class are directly coupled, including a bipartite (two-class) example and a three-class example containing an odd cycle.

[0014] FIG. 3 is a schematic diagram of a node circuit in accordance with some embodiments, comprising input gating, a weighted summation network implemented as a population count circuit for binary weights, a comparator implementing the threshold function, an output stage, and an optional inertial delay element that suppresses hazard-induced pulses.

[0015] FIG. 4 is a timing diagram illustrating a delay-differentiated embodiment in which a second update class is associated with a signal propagation delay exceeding the settling time of a first update class, producing an emergent alternating update schedule without a clock.

[0016] FIG. 5 is a block diagram of a handshake-sequenced embodiment comprising per-class completion detection circuitry and a state-holding phase element implemented as a generalized Muller C-element, having a set input responsive to completion of one update class and a reset input responsive to completion of the other, the phase output and its complement constituting the class enable signals.

[0017] FIG. 6 is a schematic diagram of convergence detection circuitry that asserts a convergence signal when a complete pass through all update classes produces no output transitions.

[0018] FIG. 7 is a flowchart of a method of producing a self-timed threshold-logic circuit from a trained recurrent network, including sparsification, retraining on a sparse support, vertex coloring, sequencing assignment, and logic synthesis.

[NOTE TO PATENT AGENT: Formal drawings to be prepared from the figure descriptions above and the detailed description below. Informal inventor sketches to be supplied.]

DETAILED DESCRIPTION

[0019] Reference will now be made to certain embodiments, examples of which are illustrated in the accompanying drawings. The described embodiments are illustrative and are not intended to limit the scope of the claims. As used herein, the singular forms are intended to include the plural forms as well, unless the context clearly indicates otherwise. The term “coupled” refers to direct or indirect electrical connection. As used herein, a “clock signal” means any free-running periodic timing signal, of any phase or frequency, used to control the timing of state changes; a plurality of phase-shifted periodic signals constitutes a plurality of clock signals. Circuits described herein as clockless or self-timed employ no clock signal: the update ordering is established by signal propagation and settlement, or by completion events, rather than by any periodic timing reference. A node circuit is said to respond, and its output to attain a settled value, when the output assumes the value of the node's threshold function evaluated on settled inputs and persists at that value; transient pulses that do not persist beyond a settling interval, including hazard-induced pulses, constitute neither a response nor a settled value.

Network Model and Terminology

[0020] A recurrent threshold-logic network comprises N node circuits indexed i = 1, ..., N. Each node circuit i maintains a binary output V(i) taking values in {0, 1} and receives as inputs the outputs of a set of other node circuits, referred to as the neighbors of node i. Each pair of coupled nodes (i, j) is associated with a coupling weight W(i,j), and each node with a threshold T(i). Node i evaluates the Boolean threshold function V(i) = 1 if the sum over neighbors j of W(i,j) times V(j) is greater than or equal to T(i), and V(i) = 0 otherwise. A Boolean threshold function is a linearly separable Boolean function and is realizable as combinational logic; accordingly, each node circuit is a combinational logic block, and the network as a whole, being recurrently interconnected combinational logic without registers or a clock, constitutes an asynchronous sequential circuit whose stable states are the fixed points of the network dynamics.

[0021] Networks specified in a signed convention, in which node states s(i) take values in {−1, +1} as in Ising-model formulations, are converted to the {0, 1} convention by the substitution s = 2V − 1, with the weights retained and each threshold adjusted by half the sum of the incident weights. Both conventions describe the same dynamics, and the claims are intended to cover implementations of either convention.

[0022] The interaction graph G of the network has one vertex per node circuit and an edge between vertices i and j whenever W(i,j) is nonzero. For symmetric weights, W(i,j) = W(j,i), the network possesses an energy function that is non-increasing under any serialized schedule of single-node updates, and every such schedule terminates at a fixed point. Under fully simultaneous update of all nodes, however, the network may enter a period-two oscillation. The present disclosure resolves this tension by concurrent evaluation within independent sets and enforced ordering between them.

Partition into Update Classes by Vertex Coloring

[0023] The node circuits are partitioned into K update classes C(1), ..., C(K) such that no edge of the interaction graph connects two vertices in the same class; that is, each class is an independent set and the partition is a proper vertex coloring of G. Because nodes within a class share no edge, no node in a class reads the output of any other node in the same class, and all nodes of a class may evaluate simultaneously with an aggregate effect identical to evaluating them one at a time. Provided that classes are updated in an ordering in which each class observes settled values from the other classes, the network evolution is a block-sequential schedule, and for symmetric weights the energy function decreases with every state change until a fixed point is reached. Oscillation of the kind afflicting fully parallel synchronous update cannot occur.

[0024] The number of classes required equals the chromatic number of the interaction graph, which for a sparse graph of maximum degree D is at most D + 1 and is typically much smaller. The coloring need not be optimal; a greedy coloring computed during a configuration or compilation step is sufficient. In some embodiments the coloring is computed once for a fixed trained network and is reflected statically in the circuit structure; in other embodiments, for example reconfigurable fabrics, the class assignment is programmable.

[0025] Two update classes suffice when the interaction graph is bipartite. In some embodiments, bipartite structure is obtained by constraint: the coupling topology is restricted during training such that nodes are divided into two layers with couplings only between layers. In other embodiments, bipartite structure is obtained by transformation: each network variable is represented by a pair of duplicate node circuits, one in each class, coupled to one another by an agreement-enforcing (ferromagnetic) weight of sufficient magnitude that the duplicates agree in every stable state, while all other couplings incident on the variable are divided between the duplicates such that every coupling connects nodes of different classes. This transformation renders any interaction graph bipartite at a cost of doubling the node count and is advantageous when a two-phase sequencing structure is preferred.

Node Circuit Embodiments

[0026] FIG. 3 illustrates a node circuit in accordance with some embodiments. Inputs from neighbor nodes are combined by a weighted summation network followed by a comparison against the threshold. For general integer weights, the summation network comprises an adder tree in which each input is gated by its weight magnitude and sign; the comparator output constitutes the node output. In embodiments in which the weights are binary (0 or 1) or ternary (−1, 0, +1), the summation reduces to one or two population count (popcount) circuits: the number of asserted inputs with positive weight, less the number of asserted inputs with negative weight, is compared against the threshold. Population count circuits are efficiently realized as compressor trees, and the node circuit cost then grows only with node degree, making sparse, low-precision networks particularly economical. In further embodiments, the threshold function is realized as a network of majority gates, or as a capacitive or otherwise mixed-signal threshold gate presenting a digital output; the claims are not limited to a particular realization of the threshold function.

[0027] A node circuit optionally includes an enable input by which the sequencing circuitry gates whether the node's output stage may transition. In delay-differentiated embodiments described below, the enable input may be absent and sequencing is effected through propagation delay alone. In handshake-sequenced embodiments, the enable input is driven by the handshake circuitry.

Sequencing Without a Global Clock: Delay-Differentiated Embodiment

[0028] In a first family of embodiments, illustrated in FIG. 4, sequencing circuitry comprises delay circuitry establishing different signal propagation delays for different update classes. Delay elements, for example buffer chains, current-starved buffers, or programmable delay lines, are placed in the input paths, the output paths, or within the node circuits of each class, such that the effective evaluation delay d(k) of class k differs among classes. In a two-class embodiment, the delay d(2) of the second class is selected to exceed the worst-case settling time of the first class, the settling time comprising the evaluation delay of the first class together with interconnect delay and any internal ripple time. Under this condition, whenever the second class responds to a transition, the first class has already settled, and conversely the first class, being fast, settles between successive transitions of the slow class. An alternating, block-sequential schedule thereby emerges from the delay structure itself, without any clock, sequencer, or state machine.

[0029] In some embodiments the delay elements are programmable, and the delay ratio is calibrated after manufacture, for example by a built-in self-test procedure that increases the ratio until oscillation is no longer observed at a worst-case operating corner, with a guard band applied. Programmability mitigates the sensitivity of delay-based sequencing to process, voltage, and temperature variation. Delay-differentiated sequencing offers minimal area overhead and is suited to applications in which operating conditions are controlled or in which occasional ordering violations are tolerable as stochastic perturbations, as discussed below in connection with annealing.

[0030] The delay elements may exhibit transport delay characteristics, in which any input feature, however narrow, is reproduced at the output after the delay interval, or inertial delay characteristics, in which pulses of duration shorter than a rejection window are suppressed rather than propagated. In embodiments in which the delay element associated with a node circuit or update class is inertial, with a rejection window not less than the disturbance interval of an evaluation phase, comprising the settling window of the preceding update class together with the internal settling time of the node circuit's combinational logic, transient hazard pulses generated within the combinational logic during the arrival of input changes are suppressed by the delay element, and the node output makes at most one transition per evaluation phase, to the value of the threshold function evaluated on settled inputs. The delay element thereby serves simultaneously as sequencing means and as hazard filtering means, and hazard-free logic synthesis of the threshold functions is not required. Inertial delay elements may comprise a resistive-capacitive stage coupled to a hysteretic buffer, a current-starved buffer, or a digital pulse filter in which a signal is combined with a delayed replica of itself such that only features persisting longer than the replica delay propagate. In some embodiments the pulse filter is constructed from logic cells of the same type as the node circuit logic, whereby the rejection window tracks the settling time of the logic across process, voltage, and temperature variation. If operating conditions cause the disturbance interval to exceed the rejection window, escaping transitions constitute transient perturbations of the network state that are absorbed by the attractor dynamics as described herein, such that the circuit degrades toward stochastic convergence rather than malfunction.

Sequencing Without a Global Clock: Completion-Detected Handshake Embodiment

[0031] In a second family of embodiments, illustrated in FIG. 5, the update ordering is enforced by causality rather than by timing margin. Completion detection circuitry associated with each update class asserts a completion signal when the node circuits of that class have settled. Handshake circuitry receives the completion signals and enables evaluation of each update class in response to completion of the preceding class in a cyclic order, for example by a loop of Muller C-elements in a two-phase or four-phase signaling discipline. Because a class is enabled only upon completion of its predecessor, the block-sequential ordering holds by construction across all process, voltage, and temperature conditions, and the circuit is amenable to formal verification using established asynchronous design methodologies.

[0032] In some embodiments with exactly two update classes, the handshake circuitry comprises a single state-holding phase element implemented as a generalized Muller C-element, that is, a set-reset latch whose set and reset excitation functions are derived from the completion signals: the set function is responsive to assertion of the completion signal of the second update class and the reset function is responsive to assertion of the completion signal of the first update class, with each excitation function optionally conjoined with the complement of the opposing completion signal to guarantee mutually exclusive excitation. A phase signal produced by the phase element directly constitutes the enable signal of the first update class, and its complement constitutes the enable signal of the second update class. Sequencing under a four-phase return-to-zero discipline is obtained by generating each completion signal from the corresponding enable signal, such that deassertion of an enable deasserts the associated completion signal before the opposing excitation can assert. For more than two update classes, the phase element generalizes to a one-hot ring of state-holding elements, the set function of each element being responsive to completion of the preceding update class in the cyclic order and the reset function being responsive to completion of its own update class, a single token circulating through the ring.

[0033] The completion signals may be generated by any of several mechanisms. In a bundled-data embodiment, each completion signal is produced by passing the corresponding enable signal through a matched delay line constructed from logic cells of the same type as the update class's combinational logic and sized to exceed the worst-case settling time of that class, whereby the matched delay tracks the logic delay across process, voltage, and temperature variation and the timing assumption is local to each completion generator. In a delay-insensitive embodiment, node outputs are dual-rail encoded and per-class completion trees detect validity of all node outputs, eliminating timing assumptions at the cost of approximately doubled datapath wiring. In a further embodiment, transition detectors on node outputs, combined with a bounded quiescence interval, generate the completion signals. Combining the enable-gating described below with any of these mechanisms preserves the single-input-change property of node evaluation.

[INVENTOR FINALIZATION — remaining selections for the phase-control embodiment: (a) matched-delay sizing methodology and guard-band margin; (b) confirmation of four-phase versus two-phase signaling; (c) power-on reset and initialization protocol for the phase element and node state. To be specified per the inventors' final design.]

[0034] A further advantage of handshake sequencing is that it permits deterministic, glitch-free node evaluation using only single-input-change hazard-free logic. In embodiments in which each node circuit of an update class comprises an enable input asserted by the handshake circuitry only after the completion signal of the preceding update class, all data inputs of the node circuit are static at the moment the enable input is asserted, and the enable input is therefore the sole changing input during evaluation. Every evaluation is thereby rendered a single-input-change event by construction, notwithstanding that multiple neighbor outputs may have changed during the preceding phase. Combinational logic that is hazard-free with respect to a single input change then suffices to guarantee the absence of hazard-induced output glitches. For threshold functions this property is obtained at little cost: a threshold function is unate in each literal when negative-weight inputs are expressed in complemented form, and a sum-of-products cover comprising all prime implicants of a unate function, which coincides with its unique irredundant cover, is free of static hazards for single input changes without additional consensus terms. Embodiments not employing enable gating may instead rely on the attractor dynamics of the network to tolerate hazard-induced glitches as stochastic perturbations, as described elsewhere herein; hazard-induced perturbation cannot corrupt a converged state, since a quiescent network exhibits no input transitions from which a hazard could arise.

[0035] In some embodiments, completion detection is hierarchical, with regional completion trees whose outputs are combined by a class-level C-element or AND tree, limiting wiring overhead. In embodiments with exactly two update classes, the handshake circuitry reduces to a single pair of completion trees and a small number of sequencing gates, and the overhead of handshake sequencing is modest relative to the node array.

Hazard Considerations

[0036] Multi-level combinational realizations of threshold functions are subject to logic hazards, and evaluation phases in which multiple inputs of a node circuit change, some increasing and some decreasing the weighted sum, are subject to function hazards that cannot be eliminated by any logic realization. Accordingly, the circuits described herein do not depend on hazard-free logic synthesis. In the delay-differentiated embodiments, hazard-induced pulses are suppressed by inertial delay elements as described above, or alternatively are tolerated as stochastic perturbations absorbed by the attractor dynamics. In the handshake-sequenced embodiments with enable gating, node evaluation is a single-input-change event by construction, and logic that is hazard-free with respect to a single input change, obtainable for threshold functions at low cost from unate covers as described above, suffices for deterministic glitch-free operation. Hazard-freedom with respect to arbitrary multiple-input changes is neither required nor, in general, attainable.

Convergence Detection

[0037] A fixed point of the network corresponds to electrical quiescence: no node output transitions. In clockless operation, dynamic power consumption is therefore proportional to update activity and falls substantially to zero at convergence, in contrast to clocked implementations that continue to expend clocking and registering energy after the state has settled. Convergence detection circuitry, illustrated in FIG. 6, exploits this property to provide an explicit done indication. In handshake-sequenced embodiments, transition detectors on the node outputs of each class are sampled at each class completion; if a complete cyclic pass through all K update classes completes with no transition detected in any class, the convergence signal is asserted. For symmetric coupling weights and block-sequential ordering, assertion of the convergence signal is guaranteed to occur in finite time, since the energy function is integer-valued on a bounded range and strictly decreases with every state change. In delay-differentiated embodiments, the convergence signal is generated by an activity monitor asserting after an interval of quiescence exceeding the slowest class delay by a configured margin.

Configuration and Training Methods

[0038] FIG. 7 illustrates a method of producing a circuit as described above from a trained recurrent network. Trained coupling weights are obtained. The weight matrix is sparsified, for example by magnitude pruning, to a target degree bound. The weights are then retrained subject to the sparse connectivity support: in some embodiments, each node's incoming weights and threshold are re-fit by a perceptron-type learning procedure restricted to the surviving connections, restoring storage or solution quality lost to pruning without altering the support. Optionally, training is additionally constrained to yield a bipartite interaction graph, or the node-duplication transformation described above is applied. A proper vertex coloring of the resulting interaction graph is computed, for example greedily. Sequencing is assigned, either by associating a delay value with each class or by instantiating per-class completion detection and handshake circuitry. The node threshold functions are synthesized to a gate-level netlist, which is placed and routed to produce the circuit. The method may be embodied in electronic design automation software executed by a computer, producing a circuit definition dataset from which the circuit is manufactured or with which a field-programmable device is configured.

[0039] In reconfigurable embodiments, the network is realized on a field-programmable gate array or similar fabric operated without registered feedback, the lookup tables implementing the threshold functions and the class delays or handshake circuitry implemented in fabric resources; the weight values, coloring, and sequencing assignment constitute a configuration loadable onto the fabric, permitting different networks to be realized on the same device.

Applications

[0040] In associative memory embodiments, stored patterns are encoded in the coupling weights, an input pattern is loaded as the initial network state through initialization circuitry, the network is released to evolve under the sequencing described above, and upon assertion of the convergence signal the node outputs are read as the recalled pattern. Because switching activity, and hence energy consumption, is proportional to the work of convergence, the disclosed circuits are suited to energy-constrained applications including embedded and edge devices.

[PLACEHOLDER — EXPERIMENTAL EXAMPLES: Insert prophetic or actual example paragraph(s) when experimental results are available from the co-inventor's work, e.g., network sizes, degree bounds after sparsification and retraining, number of colors obtained, convergence times, and comparison against a clocked baseline. Actual results strengthen enablement but are not required for filing.]

Concluding Statements

[0041] While particular embodiments have been described, the disclosure is not limited to the embodiments described, and modifications and variations will be apparent to those of ordinary skill in the art. Features described in connection with one embodiment may be combined with features of other embodiments. The threshold functions may be realized in any logic family; the sequencing circuitry may combine delay-based and completion-detected mechanisms; the number of update classes may be any integer greater than one; and the network may be realized monolithically, across chiplets, or on reconfigurable fabric. The scope of the invention is defined by the appended claims and their equivalents.

[NOTE TO PATENT AGENT — INFORMATION DISCLOSURE STATEMENT: The following references were identified in a preliminary (non-professional) prior art search and appear material; include in the IDS per 37 C.F.R. 1.56 and supplement with a professional search: (1) Aadit et al., “Massively parallel probabilistic computing with sparse Ising machines,” Nature Electronics 5, 460 (2022) (arXiv:2110.02481) — graph-colored p-bit groups driven by phase-shifted periodic clocks; closest art; distinguished as clocked, open-loop, stochastic. (2) Aadit et al., “All-to-all reconfigurability with sparse and higher-order Ising machines,” Nat. Commun. (2024) — colored clocks, master-graph multiplexing. (3) Sutton et al., “Autonomous probabilistic coprocessing with petaflips per second” (arXiv:1907.09664) — clockless, sequencerless p-bits with NO enforced ordering. (4) U.S. Patent 11,966,716 (Hitachi) — duplicated-spin (PCA) bipartite transformation for fully parallel clocked annealing; material to dependent claim on node duplication. (5) U.S. Patents 11,817,859; 11,716,083-086; 11,658,664; 11,652,482/487 (Intel) — asynchronous circuits employing capacitive threshold/majority gates; material to node-circuit dependent claims. (6) Rosin et al., “Excitability in autonomous Boolean networks” (arXiv:1208.6181) — unclocked recurrent Boolean gate networks on FPGA, no colored-partition ordering. (7) Hopfield, PNAS 79:2554 (1982). (8) Kosko, “Bidirectional associative memories,” IEEE Trans. SMC 18:49 (1988) — bipartite alternating update. (9) Goles et al., block-sequential update literature. (10) Orponen, “Computing with truly asynchronous threshold logic networks,” Theor. Comput. Sci. 174:123 (1997).]

CLAIMS

What is claimed is:

1. An integrated circuit comprising: a plurality of node circuits, each node circuit configured to produce a binary output according to a Boolean threshold function of a plurality of binary inputs; a recurrent interconnection network coupling outputs of the node circuits to inputs of other node circuits so as to define an interaction graph containing at least one cycle, wherein the plurality of node circuits is partitioned into a plurality of update classes such that no two node circuits belonging to a same update class are directly coupled to one another in the interaction graph; and sequencing circuitry configured to enforce, in the absence of any clock signal, an update ordering among the update classes in which a settled output value of a node circuit of a second update class is responsive to outputs of node circuits of a first update class only after the node circuits of the first update class have settled to stable values, the update ordering being conditioned on signal propagation and settlement rather than on any periodic timing reference.

2. The integrated circuit of claim 1, wherein the sequencing circuitry comprises delay circuitry establishing different signal propagation delays for different update classes, a propagation delay associated with the second update class exceeding a settling time of the first update class.

3. The integrated circuit of claim 2, wherein the delay circuitry comprises programmable delay elements configured to be calibrated after manufacture.

4. The integrated circuit of claim 1, wherein the sequencing circuitry comprises: completion detection circuitry configured to assert, for each update class, a completion signal indicating that the node circuits of that update class have settled; and handshake circuitry configured to enable updating of each update class responsive to assertion of the completion signal of a preceding update class in a cyclic order.

5. The integrated circuit of claim 4, wherein the handshake circuitry comprises at least one Muller C-element.

6. The integrated circuit of claim 1, further comprising convergence detection circuitry configured to assert a convergence signal upon determining that a complete pass through all of the update classes has produced no change in any node circuit output.

7. The integrated circuit of claim 1, wherein the plurality of update classes consists of exactly two update classes and the interaction graph is bipartite.

8. The integrated circuit of claim 7, wherein at least one variable of a network implemented by the integrated circuit is represented by a pair of duplicate node circuits assigned to different update classes and coupled to one another by an agreement-enforcing coupling weight, whereby the interaction graph is rendered bipartite.

9. The integrated circuit of claim 1, wherein coupling weights of the interaction graph are binary or ternary valued, and wherein at least one node circuit comprises a population count circuit configured to evaluate the Boolean threshold function of that node circuit at least in part.

10. The integrated circuit of claim 1, wherein the interaction graph is sparse, each node circuit being coupled to fewer than all other node circuits, a maximum degree of the interaction graph being bounded independently of a total number of the node circuits.

11. The integrated circuit of claim 1, wherein at least one node circuit comprises an adder network coupled to a comparator, or a network of majority gates, implementing the Boolean threshold function of that node circuit.

12. The integrated circuit of claim 1, wherein at least one node circuit comprises a multi-level combinational logic realization in which transient logic hazards are permitted to propagate to the binary output of the node circuit as stochastic perturbations of a network state.

13. The integrated circuit of claim 1, further comprising initialization circuitry configured to load an input pattern as an initial state of the node circuits, wherein coupling weights of the interaction graph encode one or more stored patterns, and wherein outputs of the node circuits at convergence constitute a recalled pattern, the integrated circuit thereby operating as a content-addressable associative memory.

14. A method of operating a recurrent network of threshold-logic node circuits, the method comprising: partitioning the node circuits into a plurality of update classes such that no two node circuits belonging to a same update class are directly coupled in an interaction graph of the network; and evaluating the network in the absence of any clock signal while enforcing, by sequencing circuitry, an update ordering among the update classes in which settled output values of node circuits of each update class are responsive to outputs of node circuits of a preceding update class only after the node circuits of the preceding update class have settled to stable values, whereby the network converges to a fixed point without oscillation attributable to simultaneous update of coupled node circuits.

15. The method of claim 14, wherein enforcing the update ordering comprises establishing different signal propagation delays for different update classes.

16. The method of claim 14, wherein enforcing the update ordering comprises detecting completion of settling of each update class and enabling a subsequent update class responsive to the detected completion.

17. The method of claim 14, further comprising asserting a convergence signal upon determining that a complete pass through all of the update classes has produced no change in any node circuit output, and reading outputs of the node circuits responsive to the convergence signal.

18. A method of producing an asynchronous logic circuit from a trained recurrent network, the method comprising: obtaining coupling weights and thresholds of the trained recurrent network; sparsifying the coupling weights to a connectivity support having a bounded degree; retraining the coupling weights and thresholds subject to the connectivity support; computing a proper vertex coloring of an interaction graph defined by the retrained coupling weights, the coloring defining a plurality of update classes in which no two directly coupled nodes share an update class; assigning to the update classes a sequencing mechanism operating without any clock signal and enforcing settlement of each update class before response of a subsequent update class; synthesizing, for each node of the network, combinational logic implementing a Boolean threshold function defined by the retrained coupling weights and threshold of that node; and producing a circuit definition dataset comprising the synthesized combinational logic and the assigned sequencing mechanism, the circuit definition dataset being usable to manufacture an integrated circuit or to configure a programmable logic device.

19. The method of claim 18, wherein retraining comprises re-fitting incoming coupling weights and the threshold of each node by a perceptron-type learning procedure restricted to the connectivity support of that node.

20. The method of claim 18, further comprising constraining the interaction graph to be bipartite, by constraining training of the recurrent network to a two-layer coupling topology or by representing each variable of the network with a pair of duplicate nodes coupled by an agreement-enforcing weight, wherein the proper vertex coloring consists of exactly two update classes.

21. The integrated circuit of claim 4, wherein each node circuit of at least one update class comprises an enable input coupled to the handshake circuitry, the handshake circuitry being configured to assert the enable input only after assertion of the completion signal of the preceding update class, whereby data inputs of the node circuit are static upon assertion of the enable input and the enable input is a sole changing input during evaluation, and wherein combinational logic of the node circuit is hazard-free with respect to a single input change, whereby evaluation of the node circuit is free of hazard-induced output glitches.

22. The integrated circuit of claim 2, wherein the delay circuitry comprises an inertial delay element configured to suppress propagation of pulses of duration shorter than a rejection window, the rejection window being not less than a disturbance interval comprising the settling time of a preceding update class, whereby hazard-induced pulses generated within combinational logic of a node circuit are suppressed and an output of the node circuit makes at most one transition per evaluation phase.

23. The integrated circuit of claim 4, wherein the handshake circuitry comprises a state-holding phase element having a set function responsive to the completion signal of one update class and a reset function responsive to the completion signal of another update class, an enable signal of at least one update class being derived from a state of the phase element.

24. The integrated circuit of claim 4, wherein the completion detection circuitry comprises, for at least one update class, a matched delay line coupled to receive an enable signal of that update class, the matched delay line being constructed from logic cells of a same type as combinational logic of that update class and having a delay exceeding a worst-case settling time of that update class, an output of the matched delay line constituting the completion signal of that update class.

ABSTRACT

A clockless circuit implements a recurrent network of Boolean threshold-logic node circuits. The node circuits are partitioned into update classes forming independent sets of the network's interaction graph, so that node circuits within a class may evaluate concurrently without interacting. Sequencing circuitry enforces, without any clock signal, an ordering in which each update class settles before another update class responds, by delay differentiation among classes or by per-class completion detection with handshake circuitry. The aggregate evolution is equivalent to a block-sequential update schedule, guaranteeing convergence to a fixed point for symmetric coupling weights while avoiding the oscillation associated with simultaneous update of coupled nodes. Convergence detection circuitry asserts a done signal upon electrical quiescence across a full pass of the classes. Embodiments include bipartite two-class networks, population-count node circuits for binary weights, inertial delay elements that suppress hazard-induced pulses, and content-addressable associative memories, together with methods of compiling trained networks into such circuits.